# 04_feature_extract.py

CELL 04 — Feature extraction (full set, NO selection here). Selection happens inside each CV fold in stage 05 to avoid leakage.

This stage turns every window into the full feature vector (invariant + baseline-deviation + cross-signal) using build_run_features from pdm_common. A key rigor point is stated in the header: NO feature selection happens here. Selecting features on the whole dataset before cross-validation would leak test information into training; instead selection is a pipeline step fitted per fold in stage 05.

## Build features for every run

for rid, g in clean.groupby("run_id"):

g = g.sort_values("time").reset_index(drop=True)

feats = P.build_run_features(g[["time"] + common], g["label"].iloc[0], rid,

bool(g["always_on"].iloc[0]), common)

if not feats.empty:

parts.append(feats)

features = pd.concat(parts, ignore_index=True)

Runs the shared build_run_features per run (baseline computed per run, then one feature row per window) and concatenates into one feature matrix. Because it reuses the exact pdm_common code, the training features are guaranteed identical to what the app computes at serving time.

## Guard: features must be finite

num = features.select_dtypes(float)

features[num.columns] = num.replace([np.inf, -np.inf], np.nan).fillna(0.0)

features.to_parquet(P.ART_DIR / "features.parquet")

The invariant design should already guarantee finite features, but this is a belt-and-braces guard: any inf/NaN (e.g. a degenerate window) is coerced to 0.0 so no downstream model chokes on a non-finite value. The clean feature matrix is cached to Parquet for stages 05 and 06.

## Sanity check — features differ by FAULT, not by run scale

show = [c for c in ["pressure_cov", "vdc_dmean", "power_cov", "corr_vdc_i"] if c in feat_cols]

print(features.groupby("label")[show].mean().round(3).to_string())

Prints per-class means of a few representative invariant features. The intent is a quick eyeball check that these features separate by FAULT CLASS, not by which run they came from. If a supposedly invariant feature instead tracked the operating point, its per-class means would look arbitrary; separation by class is the sign the invariance is doing its job.