# CHAPTER 8 – RESULTS, DISCUSSION & APPLICATION

All figures in this chapter are generated directly from artifacts/ (the

frozen two_stage_model.joblib bundle and its validation outputs) or

captured live from the running 07_streamlit_app.py dashboard — none are

mocked. Sources are named at the end of each section.

Figure folders:

docs/report_figures/ — pipeline & model-development figures (Ch. 5–6)

docs/chapter7_validation_figures/ — validation figures (Ch. 7)

docs/chapter8_gui_screenshots/ — live GUI captures for this chapter

## 8.1 Experimental Results

The deployed two-stage system was run end-to-end on all 12 usable

simulation runs (248 analysis windows total) under leave-one-run-out

cross-validation — the same protocol used throughout Chapter 7.

Table 8.1: Headline Experimental Results

| Component | Result | Basis |
| --- | --- | --- |
| Stage 1 — fault detection | 100% (239/239 fault windows) | in-run Mahalanobis threshold |
| Stage 1 — healthy false-alarm rate | 0% (0/9 healthy windows) | same |
| Stage 2 — diagnosis accuracy | 89.1% | LOGO, 221 windows, 10 folds |
| Stage 2 — macro F1 | 0.906 | same |
| Confidence gate — unseen fault (FlexibleShaft) rejected | 100% (18/18 windows) | never trained, held out entirely |
| Confidence gate — known-fault pass rate | 78.7% | out-of-fold predictions |
| Confidence gate — accuracy on passed windows | 86.8% | same |
| Deployed dashboard vs. notebook parity | 5/5 correct verdicts, 100% window agreement | headless smoke test |

Figure 8.1: End-to-End System Performance Scorecard

The system meets every obligation in its test plan (§7.1): it detects 100%

of faults, correctly names the fault in 89.1% of cases, and safely defers

("Unknown fault") on the one fault type it was never trained on, rather than

guessing.

Source: `artifacts/two_stage_metrics.json`, `artifacts/two_stage_model_comparison.csv`.

## 8.2 Performance Comparison

Seven candidate Stage-2 models were trained and validated on identical

inputs (221 windows × 51 features, 3 classes, LOGO). The deployed model is

the soft-voting ensemble of ExtraTrees + LightGBM + CatBoost.

Table 8.2: Model Comparison (Leave-One-Run-Out)

| Model | Macro F1 | Accuracy | Δ vs. deployed |
| --- | --- | --- | --- |
| Voting ensemble (ET+LGBM+CatBoost) | 0.906 | 89.1% | — |
| HistGradientBoosting | 0.902 | 88.7% | −0.004 |
| Random Forest | 0.897 | 88.2% | −0.009 |
| LightGBM | 0.887 | 87.3% | −0.019 |
| Logistic Regression | 0.872 | 85.5% | −0.034 |
| Extra Trees (standalone) | 0.820 | 80.1% | −0.086 |
| SVM (RBF) | 0.798 | 80.1% | −0.108 |

Figure 8.2: Stage 2 Model Comparison (Macro F1 & Accuracy)

The voting ensemble beats every one of its own individual members —

including Extra Trees alone, its strongest constituent (0.820 → 0.906 inside

the vote) — which is direct evidence that the ensemble's benefit comes from

combining genuinely different error patterns (random-split bagging,

leaf-wise boosting, ordered-residual boosting), not from any single strong

model.

Against the earlier flat 5-class baseline (macro F1 0.494, accuracy 68.5%,

which could not separate Healthy or FlexibleShaft at all — see §6.1),

the adopted two-stage design is a +0.412 macro F1 / +20.6 point accuracy

improvement, roughly 75% attributable to removing the two structurally

unlearnable classes from supervised training and 25% to the ensemble

upgrade.

Source: `artifacts/two_stage_model_comparison.csv`, `artifacts/model_comparison.csv`.

## 8.3 Feature Importance Analysis

Of the 51 engineered features (12 statistics × 4 channels + 3 cross-signal

features), importance was measured with built-in impurity importance from

the deployed ensemble's tree members, cross-checked with permutation

importance.

Table 8.3: Top 10 Features by Importance

| Rank | Feature | Importance | Physical meaning |
| --- | --- | --- | --- |
| 1 | current_ripple | 0.076 | Load-current oscillation depth |
| 2 | vdc_ripple | 0.073 | DC-bus oscillation depth |
| 3 | vdc_cov | 0.073 | DC-bus relative fluctuation |
| 4 | current_cov | 0.071 | Load-current relative fluctuation |
| 5 | vdc_crest | 0.069 | DC-bus spikiness |
| 6 | power_cov | 0.061 | Combined electrical stability |
| 7 | current_crest | 0.060 | Load-current spikiness |
| 8 | vac_zcr | 0.054 | Alternator AC dominant-frequency proxy |
| 9 | vac_speccen | 0.050 | Alternator AC spectral centroid |
| 10 | pressure_dmean | 0.049 | Hydraulic pressure level shift after fault |

Figure 8.3: Feature Importance Ranking

The ranking is dominated by electrical ripple/CoV features and the

pressure baseline-deviation feature — physically meaningful fault

drivers (generator waveform content, hydraulic pressure collapse), not

artifacts of the pipeline. A feature-group ablation confirms this is not

incidental: scale-invariant features alone (39 of 51) reach macro F1 0.477,

nearly matching the full 51-feature set's 0.494 in the earlier flat-model

league, while baseline-deviation features alone reach only 0.435 —

validating the deliberate design decision (§5.3) to never feed raw signal

level into the model.

Source: `artifacts/feature_importance.csv`, `artifacts/feature_group_ablation.csv`.

## 8.4 Discussion

Why the two-stage redesign was necessary. Four independent flat

5-class models (Extra Trees, Random Forest, XGBoost, SVM-RBF) all converged

on macro F1 ≈ 0.49 — when unrelated model families agree that closely, the

ceiling is in the data, not the model. Healthy had exactly one usable run,

guaranteeing 0% recall whenever it fell in the held-out fold; FlexibleShaft's

two runs did not resemble each other (standardized centroid distance 5.3),

guaranteeing 0% recall in both train/test directions. Restructuring the

problem — Stage 1 as an anomaly detector trained only on the healthy

reference, Stage 2 as a 3-class classifier trained only on classes with

multiple independent runs, gated by a confidence threshold — removed both

structurally unlearnable classes from supervised training rather than

asking a smarter model to solve an unsolvable problem.

Where the residual error lives. Per-run diagnostics (§7.6) show 9 of 10

held-out fault runs are diagnosed with 100% run accuracy; all error is

concentrated in one run, simplified_generator_fault, whose 24 windows are

unanimously misrouted to PumpDisplacement. This is traced to a

data-collection defect (a differently-configured Simulink export, standardized

centroid distance 6.4–6.7 from its sibling generator runs vs. 0.8 between

the siblings themselves) — not a modelling failure. The fix is re-exporting

that one run at the shared operating point, not retraining.

Why the confidence gate matters more than the accuracy number. A model

that is 89.1% accurate but always guesses among known classes is

unsafe for a real machine, because an unseen fault type will always be

mislabeled as something familiar. The 0.90 gate was chosen by measuring two

candidate thresholds against the same out-of-fold predictions (§7.3): the

"natural" 5th-percentile threshold (0.701) passes more known-fault windows

(95.0% vs. 78.7%) but lets every unseen FlexibleShaft window through

misclassified as a known fault; the deployed 0.90 gate accepts a lower known-fault

pass rate in exchange for 100% unseen-fault rejection. This trade —

accepting more "Unknown fault" verdicts to guarantee zero silent

misdiagnosis of a new fault type — is the correct one for a maintenance

tool, where a false "Unknown" costs an inspection but a false named-fault

costs trust in the system.

What this does and doesn't prove. Every number in this report is

Simscape-simulation-based; it should be read as an upper bound on

real-hardware performance until validated against physical sensor data —

this is the single largest scoping caveat on the whole project (see §8.10 and

main report Future Work).

## 8.5 GUI Overview

The trained model bundle is served through 07_streamlit_app.py, a

single-page Streamlit dashboard (streamlit run 07_streamlit_app.py) built

specifically to reuse the exact same decision logic validated in notebook

07_two_stage_pipeline.ipynb — no re-implementation, so there is no

train/serve skew between the reported metrics and what the GUI shows a user.

Figure 8.4: Dashboard — Initial State

The interface has three tabs (Dashboard, Window Data, Glossary)

and no sidebar; the header bar states the model identity (Two-Stage model,

ET + LGBM + CatBoost, gate 0.90, input channels) so a user always knows

which model produced the verdict on screen.

Source: live capture of the running app, `07_streamlit_app.py`.

## 8.6 Application Workflow

A user's session follows one linear path:

Upload a Simulink sensor log (.xlsx/.csv) or pick one of the

bundled sample datasets from the dropdown.

Inference runs once, cached (st.cache_data) — the file is

harmonized, cleaned, resampled, windowed, and passed through the frozen

Stage 1 + Stage 2 + gate pipeline (pdm_common.process_uploaded).

Machine status (KPI row) reports system status, diagnosis, share of

anomalous windows, and confidence — the four numbers a maintenance

operator needs first.

Alarm trend plots each window's Mahalanobis distance against the

Stage 1 threshold on a log scale, colored by verdict, next to a gauge of

the anomalous-window share.

Diagnosis shows the verdict share across all windows and "why this

diagnosis" — the top features (in robust σ units) that drove the Stage 1

decision.

Per-window timeline, event log (state transitions in/out of the

healthy baseline), and sensor signal plots (against a healthy

reference band) let a user drill from the verdict down to the raw

evidence.

Export: a verdict summary (.txt) and full window table (.csv) can

be downloaded; a Model trust panel exposes the same leave-one-run-out

validation numbers reported in Chapter 7, inside the tool itself.

This mirrors the standard industrial condition-monitoring pattern: header →

upload → KPIs → trend/gauge → diagnosis → drill-down → export — deliberately,

so the tool reads as familiar to an operator rather than as a research

notebook.

Source: `07_streamlit_app.py` structure and inline comments.

## 8.7 Backend Integration

Single shared codebase, zero duplication. Both the training notebooks

and the deployed app import the same module, pdm_common.py, for every

cleaning, harmonization, resampling, and feature-extraction step. The

dashboard does not reimplement any preprocessing logic — process_uploaded()

is the identical function path exercised in 07_two_stage_pipeline.ipynb.

Model loading. The frozen bundle artifacts/two_stage_model.joblib

(Stage 1 scaler + covariance + threshold, Stage 2 voting ensemble, class

names, feature column order, and the 0.90 gate) is loaded once via

st.cache_resource; the raw training data is never required at serve time.

Inference path (identical to the validated notebook flow):

dist  = S1_COV.mahalanobis(S1_SCALER.transform(X))       # Stage 1 distance
anom  = dist > S1_THR                                     # Stage 1 decision
proba2 = S2_MODEL.predict_proba(X)                         # Stage 2 probabilities
top_p, top_lbl = proba2.max(1), labels[proba2.argmax(1)]
verdict = "Healthy" if not anom else (top_lbl if top_p >= GATE else "Unknown fault")

Performance. All inference for an uploaded file runs exactly once

(st.cache_data); subsequent widget interactions (tab switches, filters)

only re-render already-computed results, so the dashboard stays responsive

without re-running the model bundle.

Deployment parity check. A headless smoke test pushed 5 raw logs through

07_streamlit_app.py and confirmed 5/5 correct verdicts with 100% window

agreement against the notebook's own predictions — the evidence that the

GUI is not a separate, potentially-diverging implementation.

*Source: 07_streamlit_app.py, pdm_common.py, artifacts/MODEL_CARD.md,

wiki/results.md (serving smoke test).*

## 8.8 GUI Screens and Results

Six live screenshots of the running dashboard, captured from the actual

07_streamlit_app.py process (not the earlier, superseded single-stage

GUI screenshots in docs/Gui_output.pdf, which predate the two-stage

pipeline and confidence gate).

Figure 8.5 — Generator fault, correctly diagnosed. Sample

simplifiied-generator-fault(st-0.5).xlsx (39 windows): system status "Fault

detected", diagnosis GeneratorFault at 100% confidence, 100% anomalous

windows, alarm-trend dots all above the threshold.

Figure 8.6 — Window Data tab. Per-window Stage 1 distance, verdict, and

Stage 2 confidence (progress-bar column) for all 39 windows of the same run

— confidence sits at 99.8–99.9% throughout, well above the 0.90 gate.

Figure 8.7 — Glossary tab. In-app definitions of every metric shown on

the dashboard (machine status, confidence, alarm trend, healthy range band,

event log, "Unknown fault"), so a first-time user does not need this report

open beside the tool.

Figure 8.8 — Leakage, correctly diagnosed. Sample Leakage_factor.xlsx

(9 windows): diagnosis Leakage at 100% confidence — confirms the dashboard

correctly separates fault types, not just healthy-vs-fault.

Figure 8.9 — Unseen fault correctly deferred. Sample

MildFlexible_shaft.xlsx (18 windows, never trained as a class): diagnosis

Unknown fault with the on-screen warning "doesn't confidently match a

trained fault — schedule an inspection." This is the confidence gate working

exactly as validated in §7.3/§8.4 — the system refuses to guess rather than

mislabeling an unfamiliar pattern.

*Source: live Playwright capture of streamlit run 07_streamlit_app.py,

2026-07-10.*

## 8.9 User Manual

Launching the app

streamlit run 07_streamlit_app.py

Opens at http://localhost:8501.

Step-by-step usage

On the Dashboard tab, either:

drag-and-drop / browse to a .xlsx or .csv Simulink log, or

pick a file from the Sample dataset dropdown.

Wait for the one-time cached analysis (a spinner reads "Analyzing log —

running the two-stage model …").

Read the four KPI cards under 2 · Machine status:

System status — Healthy or Fault detected.

Diagnosis — the named fault, or Healthy / Unknown fault.

Anomalous windows — % of time-slices beyond the healthy baseline.

Confidence — see the Glossary tab for the exact per-verdict definition.

Check 3 · Alarm trend — each dot is one window's distance to the

healthy baseline; anything above the dashed line is anomalous.

Check 4 · Diagnosis → "Why this diagnosis" for the specific sensor

features driving the verdict.

Scroll to Sensor signals to see the raw cleaned channels against the

healthy reference band, and Channel status to confirm all 4 expected

channels (pressure, current, vdc, vac) were found in the upload.

Use Export report to download a verdict summary or the full

per-window table.

Open the Model trust panel to see the same leave-one-run-out

validation numbers reported in Chapter 7, without leaving the tool.

Switch to Window Data for a filterable, sortable per-window table.

Switch to Glossary for plain-language definitions of every metric.

Input requirements. The uploaded file must contain a time column, the

four trained channels (pressure, current, vdc, vac), and at least 3

complete 0.02 s windows of data at or after t = 0.1 s (the fault-active

region) — the same requirement enforced during training (§5.2).

Reading "Unknown fault." This verdict is not an error — it is the system

correctly refusing to name a pattern it wasn't confidently trained on. It is

the designed (and validated, §7.3) behavior for any fault type resembling

FlexibleShaft. Treat it as "schedule an inspection," not "system failure."

Source: `07_streamlit_app.py` Glossary tab content and inline UI copy.

## 8.10 Limitations

Simulation-only validation. Every number in this report — Chapter 7's

validation and this chapter's results — comes from Simscape/Simulink

simulation, not physical sensor data. All metrics should be treated as an

upper bound until validated on real hardware; this is the dominant

real-world risk noted throughout the project (wiki/results.md).

Single healthy reference run. Stage 1's healthy baseline is fit on 9

windows from one run. Its false-alarm rate is validated only in-run; a

healthy machine at a different but legitimate operating point could be

flagged anomalous simply for being outside that one run's observed range

(surfaced directly in the GUI's "Healthy baseline caution" glossary

entry). More independent healthy runs would widen and de-risk this

baseline.

One data-collection defect drives all residual Stage-2 error. The

simplified_generator_fault run (24/24 windows misrouted) sits at a

standardized centroid distance of 6.4–6.7 from its sibling generator

runs — a different Simulink export configuration, not sensor noise or a

modelling gap. Until it is re-exported at the shared operating point,

GeneratorFault recall is capped at 70.7%.

FlexibleShaft is detected, not diagnosed. With only two dissimilar

runs (standardized centroid distance 5.3 apart), FlexibleShaft could not

be trained as a supervised class at all; it is correctly flagged as

"Unknown fault" by the confidence gate, but the system cannot yet name

it. More FlexibleShaft runs at consistent operating points are required

before it can become a named Stage-2 class.

Overconfident top-bin probabilities. The calibration check (§7.6)

shows the ensemble's ≈0.99 predicted-confidence bin corresponds to only

≈0.85 observed accuracy (gap 0.260, against a 0.15 tolerance), traced to

the same divergent generator run. Displayed confidence values should be

read as a ranking signal, not a literal probability — this is stated

explicitly in the GUI's own Glossary and Model trust panel, not hidden.

No deep-learning path evaluated. An LSTM sequence-model league was

scoped but never run (TensorFlow unavailable in the execution

environment); given how close Logistic Regression (0.872 macro F1) comes

to the top tree ensembles on the engineered features, the expected

marginal benefit on this dataset size (221 fault windows) is judged

small, but this remains untested.

No hyperparameter search. All model settings are fixed, conservative

defaults for small tabular data, chosen deliberately to avoid tuning

against the same LOGO folds used for reporting (§6.5) — this trades away

any headroom a validated search might have found.

*Source: §7.6 Error Analysis, artifacts/MODEL_CARD.md "Known caveats",

wiki/results.md "Honesty notes".*

*Sources for this chapter: artifacts/two_stage_metrics.json,

artifacts/two_stage_model_comparison.csv, artifacts/model_comparison.csv,

artifacts/feature_importance.csv, artifacts/feature_group_ablation.csv,

artifacts/MODEL_CARD.md, artifacts/robustness_*, 07_streamlit_app.py,

pdm_common.py, wiki/results.md, wiki/training-and-models.md, and live

captures of the running dashboard (docs/chapter8_gui_screenshots/,

2026-07-10).*